ANALYSIS OF RECTANGULAR PLATES USING FEM

sample problem (extract and run this)

MOMENTS & REACTIONS RECT PLATES U.S. DEPT. INTERIOR BUREAU OF RECLAMATION

PAGE 7 SQUARE PLATE (a/b = 1/2) FIXED ON SIDES AND BOTTOM WITH TOP FREE

11,1,11,1,5.0,3500,0.2,4

2,2,0,2

UNIFORM LOAD

1.0,0,0

CONC LOADS

0.0,2,0

5.5,5.0,6.0

5.5,7.0,6.0

LINE LOAD

0.0,0,1

ROW,11,1,11,2.0

COMBINED LOADS

1.0,2,1

5.5,5.0,6.0

5.5,7.0,6.0

ROW,11,1,11,2.0

Program Description

The plate must be rectangular. It is divided by a grid of rows and

columns into rectangular FEM. All finite elements are of the same width

and height, but the height does not have to equal the width.

Edge conditions may be free, pinned or fixed. This applies to the entire

edge.If there is an overall axis of symmetry, make it the Y-axis ("vertical"

on paper). The edge conditions are given in this order: bottom, left, top,

right. (0 = free   1 = pinned  2 = fixed)

There are 3 basic types of loads: uniform (over entire plate), concentrated

(point) loads, and line loads.

max grid size?  I ran a 21 x 21 grid in a "pure" DOS environment.

1). MOMENTS & REACTIONS RECT PLATES U.S. DEPT. INTERIOR BUREAU OF RECLAMATION

2). PAGE 7 SQUARE PLATE (a/b = 1/2) FIXED ON SIDES AND BOTTOM WITH TOP FREE

3). 11,1,11,1,5.0,3500,0.2,4

4). 2,2,0,2

5). UNIFORM LOAD

6). 1.0,0,0

7). CONC LOADS

8). 0.0,2,0

9). 5.5,5.0,6.0

10). 5.5,7.0,6.0

11). LINE LOAD

12). 0.0,0,1

13). ROW,11,1,11,2.0

14). COMBINED LOADS

15). 1.0,2,1

16). 5.5,5.0,6.0

17). 5.5,7.0,6.0

18). ROW,11,1,11,2.0

$ denotes string; except for main & sub titles, use no embedded commas

1). $Main title

2). $Sub title

3). 11 number of grid rows   1 vertical row spacing (ft)

11                cols   1 horizontal col spacing (ft)

5.0 plate thickness (inches)  {decimal place for clarity}

3500 plate mod. elas. (ksi)   0.2 plate Poisson's Ratio

4 number of load cases

4), 2,2,0,2  Edge conditions (as noted above)

5). $title of first load case

6). This order is kept for all load cases:

1.0 magnitude of uniform load (kips/sq. ft.) {this value can be zero}

0 (integer) number of concentrated loads

0 (integer) number of line loadings

7). $title of second load case     [it could have been "rear truck wheels"]

8).   0.0 magnitude of uniform load (kips/sq. ft.)

2 (integer) number of concentrated loads

0 (integer) number of line loadings

9). for each conc. load:

5.5 row position (need not be an integer)

5.0 col position          "

6.0 magnitude of load in kips

10). 5.5 7.0 6.0  for second concentrated load

11). $title of third load case    [ it could have been "wall"]

12).   0.0 magnitude of uniform load (kips/sq. ft.)

0 (integer) number of concentrated loads

1 (integer) number of line loadings

13). $ ROW or COLUMN

11 row number (if COLUMN then column number)

1  column at start of load  (if $ = COLUMN, then row at start of load)

11 column     end

2.0 magnitude of load in kips/foot

14). thru 18). 3 loads combined into one